Nuvance HOSPITAL

NETWORK INFRASTRUCTURE

Network Design & Technical Documentation

Author: Thomas Marvin

MS Cybersecurity  University at Albany Expected May 2027

Security Officer  Nuvance Health

Tool: Cisco Packet Tracer

Version: 1.0  |  Status: In Progress

# 1. Executive Summary

This document provides a comprehensive technical design specification for a large-scale, enterprise grade hospital network infrastructure. The design is built in Cisco Packet Tracer and simulates the type of network environment found in a modern multi-building hospital system, based on real-world operational experience at Nuvance Health.

The network is designed around three core principles:

- Security through segmentation HIPAA-mandated separation of clinical, administrative, guest, and building management systems

- Availability through redundancy dual core switches and fiber backbone ensure no single point of failure

- Scalability through structured design hierarchical three-tier architecture (core, distribution, access) supports future growth

The completed network spans 5 physical zones, implements 13 VLANs, and includes approximately 72 devices across routers, switches, servers, workstations, IP phones, and IoT medical devices.

# 2. Project Scope & Objectives

## 2.1 Scope

This project encompasses the full network design for a large hospital campus including:

- 1 Data Center with redundant core infrastructure

- Building A: Main Hospital 4 patient floors, ICU/OR wing, and Emergency Room

- Building B: Medical Office Building — Administration and Cafeteria/Guest

- Building C: Diagnostics & Lab Radiology, Pathology Lab, Pharmacy

- Building D: Facilities HVAC and Building Management Systems

## 2.2 Objectives

- Design a HIPAA compliant network with proper PHI isolation

- Implement VLAN based segmentation for all 13 traffic types

- Configure inter VLAN routing with restrictive ACLs

- Deploy redundant core switching to eliminate single points of failure

- Provide centralized logging via Syslog server for HIPAA audit trail requirements

- Document all configurations for portfolio and professional reference

# 3. Network Architecture

## 3.1 Design Model

The network follows the Cisco Three Tier Hierarchical Model:

- Core Layer: DC-Core-SW1 and DC-Core-SW2 (redundant Layer 3 switches)  high-speed backbone switching

- Distribution Layer: Building distribution switches (BldgA-Dist-SW, BldgC-Dist-SW)  VLAN routing, policy enforcement

- Access Layer: Floor and department switches — end device connectivity

## 3.2 Building Layout

| Zone | Building | Departments | Key Devices |
| --- | --- | --- | --- |
| Data Center | Core | IT Infrastructure | 2x Core L3 Switches, Firewall, Edge Router, 8 Servers |
| Building A | Main Hospital | Patient Floors 1-4, ICU/OR, ER | 6x Access Switches, 20+ Workstations, Phones |
| Building B | Medical Office | Administration, HR, Billing, Cafeteria | 2x Access Switches, PCs, Guest AP |
| Building C | Diagnostics | Radiology, Lab, Pharmacy | 3x Access Switches, PACS Server |
| Building D | Facilities | HVAC, Building Mgmt | 1x Access Switch, Controllers |

# 4. VLAN Design

## 4.1 VLAN Table

| VLAN ID | Name | Department / Purpose | Subnet | Gateway |
| --- | --- | --- | --- | --- |
| 10 | Clinical-Staff | Nurse stations, doctor workstations | 192.168.10.0/24 | 192.168.10.1 |
| 20 | Admin | HR, billing, front desk | 192.168.20.0/24 | 192.168.20.1 |
| 30 | IoT-Medical | Infusion pumps, patient monitors, ventilators | 192.168.30.0/24 | 192.168.30.1 |
| 40 | VoIP | IP phones hospital-wide | 192.168.40.0/24 | 192.168.40.1 |
| 50 | Guest-WiFi | Patient & visitor internet access | 192.168.50.0/24 | 192.168.50.1 |
| 60 | Radiology | PACS imaging systems, radiology workstations | 192.168.60.0/24 | 192.168.60.1 |
| 70 | Pharmacy | Medication dispensing systems | 192.168.70.0/24 | 192.168.70.1 |
| 80 | Lab | Lab analyzers, pathology systems | 192.168.80.0/24 | 192.168.80.1 |
| 90 | ER | Emergency department isolated network | 192.168.90.0/24 | 192.168.90.1 |
| 100 | ICU-OR | ICU & Operating Room — highest security | 192.168.100.0/24 | 192.168.100.1 |
| 110 | Facilities | HVAC, elevators, building management | 192.168.110.0/24 | 192.168.110.1 |
| 120 | Management | Switch/router OOB management only | 192.168.120.0/24 | 192.168.120.1 |
| 200 | DMZ | Servers (EHR, AD, DNS, DHCP, Syslog) | 10.0.0.0/24 | 10.0.0.1 |

## 4.2 VLAN Design Rationale

Each VLAN represents a distinct security zone based on data sensitivity and operational function:

- VLAN 100 (ICU-OR) is the highest security zone — devices here are directly involved in patient life-safety and must have minimal network exposure. Only EHR and Active Directory access is permitted.

- VLAN 30 (IoT-Medical) is isolated because medical IoT devices often run legacy firmware with known vulnerabilities. Segmenting them prevents lateral movement if a device is compromised.

- VLAN 50 (Guest) has no access to any internal VLAN. Patients and visitors receive internet access only through strict firewall rules.

- VLAN 120 (Management) is a dedicated out-of-band management VLAN for switch and router administration, inaccessible from all user VLANs.

- VLAN 200 (DMZ) hosts all servers with controlled inbound and outbound access enforced by the ASA firewall.

# 5. IP Addressing Scheme

## 5.1 WAN & Core Links

| Link | Network | ISP/Edge IP | Edge/Router IP |
| --- | --- | --- | --- |
| ISP to Edge Router | 203.0.113.0/30 | 203.0.113.1 | 203.0.113.2 |
| Edge Router to Firewall | 172.16.0.0/30 | 172.16.0.1 | 172.16.0.2 |
| Firewall to Core-SW1 | 172.16.1.0/30 | 172.16.1.1 | 172.16.1.2 |

## 5.2 Server IP Assignments (DMZ — 10.0.0.0/24)

| Server | IP Address | Role |
| --- | --- | --- |
| EHR-Server | 10.0.0.10 | Electronic Health Records application server |
| AD-Server | 10.0.0.11 | Active Directory — authentication & authorization |
| DHCP-Server | 10.0.0.12 | Dynamic IP assignment for all VLANs |
| DNS-Server | 10.0.0.13 | Internal DNS resolution |
| Syslog-Server | 10.0.0.14 | Centralized logging — HIPAA audit trail |
| Intranet-Server | 10.0.0.15 | Internal hospital web portal |
| Backup-Server | 10.0.0.16 | Data backup and disaster recovery |
| PACS-Server | 10.0.0.17 | Radiology imaging — PACS/DICOM storage |

# 6. Device Inventory

| Category | Device Name(s) | PT Model | VLAN / Zone | Building |
| --- | --- | --- | --- | --- |
| Edge Router | Edge-Router | Cisco 1941 | WAN | Data Center |
| Firewall | ASA-Firewall | Cisco ASA 5505 | DMZ/WAN | Data Center |
| Core Switch (x2) | DC-Core-SW1, DC-Core-SW2 | Cisco 3560 L3 | All VLANs | Data Center |
| Dist Switch | BldgA-Dist-SW | Cisco 3560 L3 | Trunk | Building A |
| Dist Switch | BldgC-Dist-SW | Cisco 3560 L3 | Trunk | Building C |
| Access Switch (x4) | F1/F2/F3/F4-Access-SW | Cisco 2960 | VLAN 10, 40 | Building A |
| Access Switch | ICU-SW | Cisco 2960 | VLAN 100 | Building A |
| Access Switch | ER-SW | Cisco 2960 | VLAN 90, 40 | Building A |
| Access Switch | Admin-SW | Cisco 2960 | VLAN 20 | Building B |
| Access Switch | Cafe-SW | Cisco 2960 | VLAN 50 | Building B |
| Access Switch | BldgB-SW | Cisco 2960 | Trunk | Building B |
| Access Switch | Radiology-SW | Cisco 2960 | VLAN 60 | Building C |
| Access Switch | Lab-SW | Cisco 2960 | VLAN 80 | Building C |
| Access Switch | Pharmacy-SW | Cisco 2960 | VLAN 70 | Building C |
| Access Switch | Facilities-SW | Cisco 2960 | VLAN 110 | Building D |
| Servers (x8) | See Section 5.2 | Server-PT | DMZ/VLAN 60 | Data Center |
| Workstations (x28) | Floor PCs, Admin PCs, etc. | PC-PT | Various | All Buildings |
| IP Phones (x12) | F1-F4 Phones, ER, Admin | IP Phone-PT | VLAN 40 | All Buildings |
| Access Point | Cafe-AP | AccessPoint-PT | VLAN 50 | Building B |
| Guest Laptops (x2) | Guest-Laptop1/2 | Laptop-PT | VLAN 50 | Building B |

# 7. Security Policies & ACLs

## 7.1 Access Control Policy

The following access control rules are implemented using extended ACLs on the Core Layer 3 switch and enforced at the distribution layer:

| Rule # | Source VLAN | Destination | Action | Justification |
| --- | --- | --- | --- | --- |
| 1 | VLAN 50 (Guest) | All internal VLANs | DENY | Guests must have zero access to clinical or admin systems |
| 2 | VLAN 30 (IoT) | All except Syslog | DENY | Medical devices are isolated — logs only for monitoring |
| 3 | VLAN 110 (Facilities) | All clinical VLANs | DENY | HVAC/BMS has no legitimate clinical network need |
| 4 | VLAN 90 (ER) | VLAN 100 (ICU) | DENY | ER and ICU remain operationally separate |
| 5 | VLAN 100 (ICU) | All except EHR + AD | DENY | Strictest zone — minimal exposure for life-safety systems |
| 6 | VLAN 10 (Clinical) | EHR-Server (10.0.0.10) | PERMIT | Clinical staff require EHR system access |
| 7 | VLAN 20 (Admin) | AD, Intranet Servers | PERMIT | Admin requires directory and portal access |
| 8 | Any | Syslog-Server (10.0.0.14) | PERMIT | All devices must send logs for HIPAA audit trail |
| 9 | Any | DNS-Server (10.0.0.13) | PERMIT | All VLANs need DNS resolution |
| 10 | All VLANs | Internet (via Firewall) | INSPECT | ASA performs stateful inspection on all outbound traffic |

## 7.2 Firewall Policy (ASA 5505)

- Outside zone (Internet) — DENY all inbound by default, permit only established return traffic

- DMZ zone — PERMIT inbound on specific ports only (443 for EHR, 389 for LDAP, 514 for Syslog)

- Inside zone — PERMIT outbound to internet with stateful inspection, deny any to outside DMZ

- Guest zone — PERMIT outbound internet only, DENY all internal destinations

# 8. HIPAA Compliance Mapping

## 8.1 HIPAA Security Rule — Technical Safeguards

| HIPAA Standard | 45 CFR Reference | Network Control Implemented |
| --- | --- | --- |
| Access Control | 164.312(a)(1) | VLAN segmentation + ACLs restrict access to EHR to Clinical VLAN only. AD authentication required for all logins. |
| Audit Controls | 164.312(b) | Syslog server captures all inter-VLAN traffic. All devices configured to forward logs to 10.0.0.14. |
| Integrity Controls | 164.312(c)(1) | PACS server on isolated VLAN 60 — imaging data cannot be modified from other VLANs. |
| Transmission Security | 164.312(e)(1) | VoIP isolated to VLAN 40 to prevent eavesdropping on clinical communications. Guest VLAN has no access to PHI traffic. |
| Person Authentication | 164.312(d) | Active Directory server provides centralized authentication for all VLAN 10, 20, 60, 80, 100 users. |

# 9. Redundancy & Disaster Recovery

## 9.1 Network Redundancy

- Dual core switches (DC-Core-SW1 and DC-Core-SW2) connected via uplink — if one fails, the other takes over

- Spanning Tree Protocol (STP) configured on redundant links to prevent loops while maintaining failover capability

- All building distribution switches connect to BOTH core switches for dual-path redundancy

## 9.2 Data Backup

- Backup-Server (10.0.0.16) performs nightly backups of EHR-Server, PACS-Server, and AD-Server

- Backup VLAN traffic isolated to prevent backup jobs from saturating clinical network segments

- Recovery Time Objective (RTO): Under 4 hours for critical clinical systems

- Recovery Point Objective (RPO): 24 hours maximum data loss tolerance

# 10. Testing & Verification Plan

## 10.1 Connectivity Tests

| Test | Source | Destination | Expected Result |
| --- | --- | --- | --- |
| Clinical to EHR | F1-NursePC1 (VLAN 10) | EHR-Server (10.0.0.10) | PASS — ping success |
| Guest isolation | Guest-Laptop1 (VLAN 50) | EHR-Server (10.0.0.10) | FAIL — ACL blocks, expected |
| IoT isolation | MedDevice-1 (VLAN 30) | Admin-PC1 (VLAN 20) | FAIL — ACL blocks, expected |
| ICU restriction | ICU-PC1 (VLAN 100) | Pharmacy-SW (VLAN 70) | FAIL — ACL blocks, expected |
| VoIP connectivity | F1-Phone1 (VLAN 40) | F2-Phone1 (VLAN 40) | PASS — same VLAN |
| DNS resolution | Any VLAN | DNS-Server (10.0.0.13) | PASS — permitted all VLANs |
| Syslog logging | Any VLAN | Syslog-Server (10.0.0.14) | PASS — permitted all VLANs |
| Core redundancy | Any device | Any server (failover test) | PASS — traffic reroutes via SW2 |

# 11. Configuration Index

Full CLI configurations for each device are stored in the /configs directory of this repository. Each file contains the complete running configuration exportable from Cisco IOS.

| Config File | Device | Key Configurations |
| --- | --- | --- |
| Edge-Router.txt | Edge-Router (1941) | NAT, default route, WAN interface |
| ASA-Firewall.txt | ASA-Firewall (5505) | Security zones, ACLs, NAT, inspection policy |
| DC-Core-SW1.txt | DC-Core-SW1 (3560) | All VLANs, SVI interfaces, OSPF, ACLs |
| DC-Core-SW2.txt | DC-Core-SW2 (3560) | Redundant core, STP root backup |
| BldgA-Dist-SW.txt | BldgA-Dist-SW (3560) | VLAN trunking, inter-VLAN routing, floor uplinks |
| BldgC-Dist-SW.txt | BldgC-Dist-SW (3560) | Radiology, Lab, Pharmacy VLAN routing |
| F1/F2/F3/F4-Access-SW.txt | Floor Switches (2960) | Access ports, voice VLAN, port security |
| ICU-SW.txt | ICU-SW (2960) | VLAN 100 access, strict port security |
| ER-SW.txt | ER-SW (2960) | VLAN 90, 40 access ports |

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| ACL | Access Control List rule set applied to router/switch interfaces to permit or deny traffic |
| DHCP | Dynamic Host Configuration Protocol automatically assigns IP addresses to devices |
| DMZ | Demilitarized Zone network segment containing public-facing servers, isolated from internal networks |
| EHR | Electronic Health Record digital version of patient medical records |
| HIPAA | Health Insurance Portability and Accountability Act — US law governing PHI protection |
| OSPF | Open Shortest Path First link-state dynamic routing protocol |
| PACS | Picture Archiving and Communication System medical imaging storage and retrieval |
| PHI | Protected Health Information — individually identifiable health data regulated by HIPAA |
| STP | Spanning Tree Protocol prevents network loops on redundant switch links |
| VLAN | Virtual Local Area Network logical network segmentation on a physical switch |
| VoIP | Voice over IP transmission of voice communications over the network |